# AV Room WorkFlow

Official URL: https://services.smsu.edu/TDClient/180/Portal/KB/Article/7984/AV-Room-WorkFlow
Category: Knowledge Base > Information Technology Services > Internal > AV Room WorkFlow
Tags: projector, audio, AV, camera, doccam, smartboard, mircrophone

_____________________________________________________

## General Guidelines

Always present: Switcher box.

Do NOT attempt anything beyond normal user interaction: Any Crestron units.

Use leave-behind guides when available. 

	→ If missing, look up the manual.

Zoom audio output should be “Same as system” input is usually “Easy IP Mixer (or Decoder)”

## Power & System Reset

### Room 201 & 217 Specific “Snowflake Room”

If no audio, check breaker for PA system.

Adjust all mixer knobs to “noon” except podium mic, that will feed back with anything more than a little gain.

Cycle input: 
	Switch to Doc Cam. 
Then switch back to PC.

Switch to Doc Cam.

Then switch back to PC.

This resets the switcher to show video correctly.

Note: If the user complains that the projector is dark, it’s awaiting replacement. October 2025

### Picture & Audio Mute

Toggle "Pic Mute" on the switcher.

Some systems toggle; others require unmute using a separate button.

Audio mute: Ensure audio is not muted.

For older Extron switchers, confirm audio is plugged into the front headphone jack. The rear audio jacks on the new PCs don’t work.

### Projector

Turn off  or dim room lights.

It is NEVER the bulb. (unless it’s completely dead)

Set the device to duplicate display.

### Camera Setup

Most cameras: 
	Controlled via remote. 
Remote is locked to desk position.

Controlled via remote.

Remote is locked to desk position.

If not preset: 
	Use touch panel to set. 
Press Camera Preset 1. 
Look for blue light indicating camera sees your input.

Use touch panel to set.

Press Camera Preset 1.

Look for blue light indicating camera sees your input.

Test camera using the Camera app.

If stuck on black or blue screen: 
	Press reset button on the EasyIP mixer.

Press reset button on the EasyIP mixer.

### Document Camera (Doc Cam)

Turn on the doc cam.

If stuck in menu: 
	Unplug and replug to reset.

Unplug and replug to reset.

### SMART Board

Ensure SmartInk is installed and running.

If ink is active but not working: 
	Unplug and replug USB connection.

Unplug and replug USB connection.

### Wireless Microphones

Push “Scan” on receiver.

Open transmitter battery door.

Press “Pair” to sync with the transmitter IR pointed at the reciever.

### SHARP TVs

TVs may enter SuperSleep mode.

First, press the ON button on the control panel.

If that fails: 
	Turn the TV off then on. 
Power switch is near the power cord.

Turn the TV off then on.

Power switch is near the power cord.

Do not swap HDMI cables to a different input, this will not work with the control programming.

### Audio Troubleshooting

External Audio amp Volume Knobs: 

	Turn up if audio is low. 
Set to 12 o’clock position for baseline.

Turn up if audio is low.

Set to 12 o’clock position for baseline.

PC Audio Output: 
	Check if system is Crestron or Extron digital. 
If neither is present, use headphones to verify output.

Check if system is Crestron or Extron digital.

If neither is present, use headphones to verify output.

Microphone 
	Check if it is set to EasyIP mixer

Check if it is set to EasyIP mixer

## If Issues Persist

Inform instructor that resolution may be delayed.

Pivot  to alternative setup if needed.

Escalate ticket and assign to Cory after troubleshooting is complete.

Do NOT unplug anything from desks or switchers — only unplug from the PC.

Accessibility Review & Update Log:

- [ ] Content accuracy verified
- [ ] Color contrast checked
- [x] Image alt-text verified (screenshots auto-captioned)
- [x] Document structure and headings formatted (perfect bold, list bulletpoints and sequences preserved)