# Phone Setup Instructions (Static VLAN + .1x Provisioning)

Official URL: https://services.smsu.edu/TDClient/180/Portal/KB/Article/8592/Phone-Setup-Instructions-Static-VLAN-1x-Provisioning
Category: Knowledge Base > Information Technology Services > Internal > Phone Setup Instructions (Static VLAN + .1x Provisioning)
Tags: zoom, phone, PhysicalPhone

_____________________________________________________

Phone Setup Instructions (Static VLAN + .1x Provisioning)

1. Prepare the Phone for First-Time Setup

Connect the phone to a static VLAN port, because new phones do not support 802.1x on first boot.

	Approved static ports include:
	Soren’s ports in BA 176
TRC reimaging stations
Scott’s office ports

Soren’s ports in BA 176

TRC reimaging stations

Scott’s office ports

2. Reset the Phone to Factory Defaults

Hold down the center (OK) button on the phone.

When the reset menu appears, select “Reset to Factory Default.”

Confirm the reset.

This ensures it starts clean and pulls the correct provisioning profile.

3. Phone Automatically Pulls Configuration

Once the phone resets and detects the static VLAN connection, it will:

	Go online
Contact Zoom’s provisioning service
Pull down all assigned configurations
Download firmware if needed
Apply settings from Scott’s Zoom Phone assignment

Go online

Contact Zoom’s provisioning service

Pull down all assigned configurations

Download firmware if needed

Apply settings from Scott’s Zoom Phone assignment

The phone may restart automatically — this is normal.

Let it fully resolve without interruption.

4. Verify 802.1x Configuration

After provisioning is complete, move the phone to the .1x testing station.

Allow the phone to boot:
	If it successfully connects and registers on the .1x network → it is ready for deployment.
If it fails, the .1x configuration may not have properly applied, and you should return it to a static port to re‑provision.

If it successfully connects and registers on the .1x network → it is ready for deployment.

If it fails, the .1x configuration may not have properly applied, and you should return it to a static port to re‑provision.

5. Deploy the Phone

At the final office location:
	Plug the wall jack into the phone’s LAN port.
Plug the PC passthrough port on the phone into the user’s computer.

Plug the wall jack into the phone’s LAN port.

Plug the PC passthrough port on the phone into the user’s computer.

Confirm:
	The phone registers in Zoom
The PC receives a valid network connection
The phone displays the correct user/extension

The phone registers in Zoom

The PC receives a valid network connection

The phone displays the correct user/extension

Deployment is complete.

Accessibility Review & Update Log:

- [ ] Content accuracy verified
- [ ] Color contrast checked
- [x] Image alt-text verified (screenshots auto-captioned)
- [x] Document structure and headings formatted (perfect bold, list bulletpoints and sequences preserved)