# Planetarium Projector Configuration

Official URL: https://services.smsu.edu/TDClient/180/Portal/KB/Article/8100/Planetarium-Projector-Configuration
Category: Knowledge Base > Information Technology Services > Internal > Planetarium Projector Configuration
Tags: projector, planetarium

_____________________________________________________

The planetarium uses a projector that can only be configured internally, these are the instructions on how to enter the computer's config site.

Materials Needed.

Laptop that has charge and can connect to the Wi-Fi AND has an ethernet port

Passover cable.

Admin permissions

Step 1: Gathering Information

To Enter the config, you will need to find the IP address of the projector which by default should be 192.168.1.100, However as a backup here is the instructions to check

1A. Press the button with three stacked lines to enter the menu.

1B. Use the arrows and check button to enter "Communications."

1C. Use the arrows and the check button to enter the "LAN" tab. In this tab the line marked "Address:" is the IP you are looking for.

Step 2: Setting-up Communication

When connecting through the Passover you will need to enter through the IP you just found. However, as you are connected to the WIFI on a "DHCP Dynamic" system you are on a different Network ID than the projector. These next steps will tell you how to set it up to connect.

2A. Connect the Passover cable into your ethernet port on the laptop

2B. Type "Ethernet" in the search on your task bar and enter the settings

2C. At the top you should be connected to an ethernet that may say "unidentified network. " Click the Dropdown arrow to see it's information

2D. When you are in the ethernet page click the button marked "EDIT" in the "IP assignment" row

2E. Set it to manual and switch IPV4 to "On"

2F. Set the following settings according to the info received in step 1

IP Address: <Projector's Network ID>(.1 to .100, EXAMPLE 192.168.1.23)

Subnet Mask: <255.255.255.0>

Gateway:<Projector's Full IP>

Preferred DNS: <Projector's Full IP>

Alternate DNS: SMSU.edu

Step 3: Get to Config site

3A. Open a browser

3B. Enter http://<Projectors_IP>:9999 where you will enter the website.

3C. Ask the planetarium manager (Most likely Dr.Murphy) what the login is and help him get the required Config done!

Accessibility Review & Update Log:

- [ ] Content accuracy verified
- [ ] Color contrast checked
- [x] Image alt-text verified (screenshots auto-captioned)
- [x] Document structure and headings formatted (perfect bold, list bulletpoints and sequences preserved)