# Setting Up Airtames

Official URL: https://services.smsu.edu/TDClient/180/Portal/KB/Article/8098/Setting-Up-Airtames
Category: Knowledge Base > Information Technology Services > Internal > Setting Up Airtames
Tags: airtame

_____________________________________________________

Things Needed:

Airtame

Security cable

Ethernet adapter

Power & HDMI plug

Power strip (preferably with a USB direct connection)

Computer with Airtame installed & access to ISE

Access to Airtame settings

WAG account

Introduction

Setting up an Airtame is relatively straightforward but involves a few important tweaks. The key to its success is ensuring it can connect to its management website, Airtame Cloud. Therefore, making sure the device is set up for internet access beforehand is essential.

1. Pre-Setup

1a. Grab your Ethernet adapter and note down the MAC address.

1b. Log in to ISE01.smsu.edu.

1c. Click on the hamburger menu

1d. Go to Endpoints.

1e. Click 'Add'

1f. Add the MAC address of the Ethernet adapter.

1e. Under Static Group Assignment, set it to SMSU_IOT_Remote_Display.

2. Hardware Setup & Connections

2a. Plug the Airtame into the TV.

2b. Connect the Ethernet adapter to an Ethernet port and wait for the Airtame to turn on.

2c. Plug the USB cable into a power block or power strip. Do not plug it into the TV’s USB port, as it will not provide a stable power supply.

2d. Plug the TV into the power strip for a tidy setup.

2e. Secure the security cable and lock up the Airtame (do this after confirming the Airtame works).

3. Configure to Cloud

3a. If this is a new Airtame, it will appear without an IP or code. Follow the instructions below. If it is not new, skip to Step 4a.

3b. Name the device using this format: <Room><Device number if multiple in room>_AT.

3c. When prompted, enter the password provided by the Airtame expert.

3d. Enter the Cloud ID provided by the Airtame expert.

3di. Log in to Airtame Cloud.

3dii. Click Add Airtame and send the code to the person setting up the device.

3e. If the Airtame is out of date, have the expert click the update icon when the new Airtame enters the cloud, and wait patiently.

4. Administrative Settings

4a. After the update is complete, or if this is a previously set up device, enter the Airtame app and connect via IP or code.

4b. Click Settings under the device name and enter the admin password from the Airtame expert. Ensure the name is correct: <Room><Device number if multiple in room>_AT.

4c. Scroll down and click Show Advanced Settings.

4d. Ensure Google Cast is turned off.

4e. Turn off Miracast.

4f. Turn off Airplay.

4g. Turn on Auto-Update.

4h. Enable PIN Connect (make sure Override is turned off).

4i. Ensure Password Protect and Airtame LED are enabled.

4j. Turn off both WiFi access points.

4k. Save the settings.

5. Cloud Settings

5a. After saving the settings, enter Airtame.Cloud and find the Airtame you are setting up.

5b. Under Appearance, click Custom Text and enter this code exactly:

{{DEVICE_NAME}}

To share, go to airtame.com/go and type:{{PAIRING_CODE}}

Or type {{DEVICE_IP}} on the app.

5c. Set this to the footer panel.

5d. Enter Signage on the left panel.

5e. Click Custom Image and upload the image below. If it asks about playlists, pull from another Working Airtame.

Accessibility Review & Update Log:

- [ ] Content accuracy verified
- [ ] Color contrast checked
- [x] Image alt-text verified (screenshots auto-captioned)
- [x] Document structure and headings formatted (perfect bold, list bulletpoints and sequences preserved)