# Setting Up BrightSigns

Official URL: https://services.smsu.edu/TDClient/180/Portal/KB/Article/8108/Setting-Up-BrightSigns
Category: Knowledge Base > Information Technology Services > Internal > Setting Up BrightSigns
Tags: BrightSigns

_____________________________________________________

Things You Will Need:

BrightSign device.

Micro SD card.

Access to BrightAuthor

Network assigning documentation (for ISE settings).

URL for provisioning: https://smsu.carouselsignage.net/Provision/BrightSign.

Step 1: Plug in the BrightSign Device

Plug in the BrightSign device and ensure it is powered on.

Verify that the network is set to static to maintain its connection between restarts.

Step 2: Assign BrightSign to Kiosk VLAN

Find the device type for future reference (this will be important during setup).

Using ISE, assign the BrightSign device’s MAC address to the Kiosk_IOT by following your Network Assigning Documentation.

Step 3: Prepare the SD Card

Open the BrightAuthor application.

Log in to the BrightSign Network (BSN).

Connect your Micro SD card to your computer and format it to exFAT.

Step 4: Set Up the BrightSign Player

Go to the Admin tab in BrightAuthor

Click Setup Player.

Set the device name to the location where the player will be installed.

Enter the following App URL: https://smsu.carouselsignage.net/Provision/BrightSign

Go to the BrightSignOS Updates dropdown, select the device type (found earlier), and choose the latest release.

Save the setup to the SD card that you formatted earlier.

Step 5: Update the BrightSign Device

Insert the SD card into the BrightSign device.

Allow the BrightSign to update automatically.

Step 6: Register the Device

After the update, you will receive a registration code.

Provide this code to Shawn so he can add the device to Carousel.

Ask him to name the device the same as the location name you assigned.

Step 7: Notify Marketing

Inform Marketing that the BrightSign device is ready once the setup and registration are complete.

Accessibility Review & Update Log:

- [ ] Content accuracy verified
- [ ] Color contrast checked
- [x] Image alt-text verified (screenshots auto-captioned)
- [x] Document structure and headings formatted (perfect bold, list bulletpoints and sequences preserved)